MEC3301

Kinematics for Robotics

3DOF Robot Arm Simulation

| Field | Details |
| --- | --- |
| Course | MEC3301 — Kinematics for Robotics |
| Assessment | Portfolio Task |
| Title | 3DOF Robot Arm: IK, Trajectory & PID Control |
| Software | MATLAB |
| Workspace | 30 x 30 x 30 mm |
| Robot Config | RRR — 3 Revolute Joints (base yaw + shoulder + elbow) |
| IK Method | Direct Geometric Inverse Kinematics |
| IK Config | Elbow-Up exclusively |
| Link Lengths | L1 = 30 mm, L2 = 25 mm |
| Waypoints | For Example: P0=(0,0,0) P1=(10,10,25) P2=(25,5,25) P3=(20,25,25) P4=(5,20,25) |
| Submission | Report + Video |
| AI Use | Level 1 only — grammar/structure checking |

# 1. Introduction

The following report describes the design, simulation, and analysis of the 3DOF robotic arm for the MEC3301 Kinematics for Robotics portfolio. The purpose is to create the simulation for the robot arm that would be starting from the origin position and reach four target positions P1 = (10, 10, 25); P2 = (25, 5, 25); P3 = (20, 25, 25); P4 = (5, 20, 25) mm by using straight-line paths in the workspace with the size 30x30x30 mm.

The simulation is performed in MATLAB and involves four engineering aspects – geometry of the robot and workspace coverage, direct geometric inverse kinematics solution, straight line Cartesian interpolation and PID loop control, which creates a set of simulated motor orders for all joints.

## 1.1 Objectives

•	Create a 3DOF robot arm where the link lengths can achieve all grid locations on a chessboard workspace.

•	Create direct geometric inverse kinematics (without DH parameters) by using an elbow up approach.

•	Create straight line interpolations between five waypoints.

•	Create a simulation of a PID controller actuating the motor joints to achieve the target angles.

•	Simulate sending UART motor command packets for each control time step.

•	Test IK with round trip error analysis through forward kinematics.

1.2 Assumptions

•	Only Elbow-up configuration of IK is used for all movement cycles.

•	Bodies are rigid — flexibility and deformation do not occur.

•	The joints have ideal actuators — no play, friction, and inertia.

•	Fixed base body at Cartesian coordinates of (0, 0, 0).

•	No gravity effects and dynamic considerations.

•	Perfect encoder feedback without quantisation and measurement errors.

•	Zero latency time for control signals to reach actuator drivers.

•	PID plant is an ideal integrator, simplifying motor dynamics model.

•	Movement targets fall into 30x30x30 mm working space region.

•	Home position is located at (0,0,0); inside arm dead zone (distance less than |L1-L2| = 5 mm).

# 2. Robot Arm Design

## 2.1 Configuration

The robot uses a Revolute-Revolute-Revolute (RRR) configuration with three rotary joints. The three joints are:

- Joint 1 (theta1) — Base yaw: rotates the entire arm about the vertical Z-axis.

- Joint 2 (theta2) — Shoulder pitch: elevates the Link 1 up and down.

- Joint 3 (theta3) — Elbow pitch: bends the Link 2 relative to the Link 1.

## 2.2 Link Length Selection and Reachability Proof

The workspace is a cube of side 30 mm. A two-link arm can only reach points within a spherical shell defined by:

Inner radius (min reach) = |L1 - L2|   (arm is fully folded)

Outer radius (max reach) = L1 + L2     (arm is fully extended)

The farthest point in the workspace is the diagonal corner:

d_max = sqrt (30^2 + 30^2 + 30^2) = sqrt (2700) = 51.96 mm

The reachability sweep examines workspace points at 5 mm spacing along each axis. Therefore, the nearest non-origin grid point occurs at:

d_min = 5 mm

With L1 = 30 mm and L2 = 25 mm:

| Parameter | Value | Requirement | Status |
| --- | --- | --- | --- |
| L1 (upper arm) | 30 mm | — | — |
| L2 (forearm) | 25 mm | — | — |
| Min reach ¦L1-L2¦ | 5 mm | <= 5 mm | PASS |
| Max reach L1+L2 | 55 mm | >= 51.96 mm (d_max) | PASS |
| Workspace diagonal | 51.96 mm | < 55 mm max reach | PASS |

A complete reachability sweep is performed at 5 mm spacing throughout the 30×30×30 mm workspace. The sweep verifies that every non-origin grid point lies within the arm reachability shell bounded by the inner radius (5 mm) and outer radius (55 mm). Therefore all required workspace locations are reachable.

## 2.3 Target Waypoints

| Point | X (mm) | Y (mm) | Z (mm) | Distance from Origin | Description |
| --- | --- | --- | --- | --- | --- |
| P0 (home) | 0 | 0 | 0 | 0.00 mm | Start/home pose (dead zone) |
| P1 | 10 | 10 | 25 | 28.72 mm | First target |
| P2 | 25 | 5 | 25 | 35.71 mm | Second target |
| P3 | 20 | 25 | 25 | 40.62 mm | Third target |
| P4 | 5 | 20 | 25 | 32.40 mm | Fourth target |

# 3. Inverse Kinematics

## 3.1 Method

Geometric inverse kinematics is performed directly, not using DH parameters modeling. With the geometric method, one gets a simple, closed form, fast, and singularity-conscious solution for a 3DOF arm without additional computation required for the DH transformation matrices. The upward arm posture selection is made solely through picking the positive root in the theta3 expression.

## 3.2 Derivation

### Step 1 — Base Yaw (theta1)

Joint 1 rotates the arm to face the target in the horizontal XY plane:

theta1 = atan2(y, x)

### Step 2 — Horizontal Reach (r)

r = sqrt (x^2 + y^2)

### Step 3 — Law of Cosines Parameter (D)

D encodes the reachability of the target. If |D| > 1, the point is outside the arm's reach:

D = (r^2 + z^2 - L1^2 - L2^2) / (2 * L1 * L2)

### Step 4 — Elbow Angle (theta3)

Positive square root selects elbow-up configuration:

theta3 = atan2(sqrt (1 - D^2), D)

### Step 5 — Shoulder Angle (theta2)

theta2 = atan2(z, r) - atan2(L2*sin(theta3), L1 + L2*cos(theta3))

## 3.3 Calculated IK Results

Using the four assessment waypoints, the following joint angles were computed:

| Point | Target (x, y, z) mm | theta1 (deg) | theta2 (deg) | theta3 (deg) | D value | Reachable? |
| --- | --- | --- | --- | --- | --- | --- |
| P0 | (0, 0, 0) | 0.0000 | 0.0000 | 0.0000 | N/A | Yes (home) |
| P1 | (10, 10, 25) | 45.0000 | 10.1678 | 117.8181 | -0.4667 | Yes |
| P2 | (25, 5, 25) | 11.3099 | 0.7806 | 99.5941 | -0.1667 | Yes |
| P3 | (20, 25, 25) | 51.3402 | 0.1555 | 85.2198 | 0.0833 | Yes |
| P4 | (5, 20, 25) | 75.9638 | 3.4518 | 108.4615 | -0.3167 | Yes |

## 3.4 FK Verification

Forward kinematics was applied to each computed angle set and the resulting end-effector position compared to the original target. The FK equations are:

% Elbow joint (end of Link 1):

j1_x = L1 * cos(theta1) * cos(theta2)

j1_y = L1 * sin(theta1) * cos(theta2)

j1_z = L1 * sin(theta2)

% End-effector (end of Link 2):

theta_total = theta2 + theta3

ee_x = j1_x + L2 * cos(theta1) * cos(theta_total)

ee_y = j1_y + L2 * sin(theta1) * cos(theta_total)

ee_z = j1_z + L2 * sin(theta_total)

| Point | Target (mm) | FK Result (mm) | Round-Trip Error |
| --- | --- | --- | --- |
| P1 | (10.00, 10.00, 25.00) | (10.00, 10.00, 25.00) | 0.0000 mm |
| P2 | (25.00, 5.00, 25.00) | (25.00, 5.00, 25.00) | 0.0000 mm |
| P3 | (20.00, 25.00, 25.00) | (20.00, 25.00, 25.00) | 0.0000 mm |
| P4 | ( 5.00, 20.00, 25.00) | ( 5.00, 20.00, 25.00) | 0.0000 mm |

Maximum round-trip error = 0.0000 mm. This confirms the IK equations are algebraically correct.

# 4. Trajectory Planning

## 4.1 Straight-Line Interpolation

The task requires straight-line end-effector motion between consecutive waypoints. Parametric linear interpolation in Cartesian space achieves this exactly:

P(t) = P_start + t * (P_end - P_start)

x(t) = x_start + t * (x_end - x_start)

y(t) = y_start + t * (y_end - y_start)

z(t) = z_start + t * (z_end - z_start)

t = 0, 1/N, 2/N, ..., 1    where N = 50 steps per segment

Since the start point lies within the dead zone of the arm (distance = 0 < innerR = 5 mm), it is considered the home position but not included in the path traced. The three arm segments include P1-P2, P2-P3, and P3-P4. Join points between arm segments that occur more than once are eliminated from the position list.

## 4.2 Trajectory Segments

| Segment | From | To | Segment Length | Steps | Step Size |
| --- | --- | --- | --- | --- | --- |
| 1 | P1 (10,10,25) | P2 (25,5,25) | 15.81 mm | 50 | 0.316 mm |
| 2 | P2 (25,5,25) | P3 (20,25,25) | 20.62 mm | 50 | 0.412 mm |
| 3 | P3 (20,25,25) | P4 (5,20,25) | 15.81 mm | 50 | 0.316 mm |

Total trajectory: 3 segments, 150 interpolated positions (plus first point = 151 total). IK is solved at each position to obtain the joint angle profile for the PID controller.

# 5. PID Control Loop

## 5.1 Overview

Each of the three joints is driven by an independent PID controller simulating a servo motor drive. The controller runs at 50 Hz (dt = 0.02 s), matching common industrial servo loop frequencies.

## 5.2 PID Equations

e(t)             = theta_desired(t) - theta_actual(t)

P_term           = Kp * e(t)                         = 2.0 * e(t)

I_term           = Ki * sum( e(t) * dt )             = 0.05 * integral

D_term           = Kd * ( e(t) - e(t-1) ) / dt      = 0.8 * derivative

u(t)             = P_term + I_term + D_term          [deg/s, clamped +/-400]

theta_actual(t+1)= theta_actual(t) + u(t) * dt

| Parameter | Value | Role |
| --- | --- | --- |
| Kp (Proportional) | 2.0 | Fast response to angle error |
| Ki (Integral) | 0.05 | Eliminates steady-state offset |
| Kd (Derivative) | 0.8 | Dampens overshoot and oscillation |
| dt | 0.02 s | 50 Hz control loop frequency |
| Rate Limit | 400 deg/s | Prevents unrealistic motor commands |

## 5.3 Motor Command Transmission

At every control step, the computed joint command is formatted into a serial packet and passed to the sendMotorCommand() function. The packet format is:

<J1:angle, J2:angle, J3:angle>

Packet transmission on actual physical hardware can be done by using UART/Serial with fprintf(serialPort, packet). In this simulation exercise, packet creation is shown by building up the string representation of the packet in function form and printing it on the command line at each 20 steps. Sample output from the simulation follows:

| Step | Time (s) | u_J1 (deg/s) | u_J2 (deg/s) | u_J3 (deg/s) | Serial Packet |
| --- | --- | --- | --- | --- | --- |
| 20 | 0.380 | ~45.0 | ~10.2 | ~117.8 | <J1:34.78,J2:7.82,J3:115.13> |
| 40 | 0.780 | ~12.1 | ~5.3 | ~85.2 | <J1:25.42,J2:4.95,J3:109.89> |
| 100 | 1.980 | ~51.3 | ~0.2 | ~99.6 | <J1:41.61,J2:1.21,J3:93.18> |

# 6. Pseudocode

## 6.1 Main Program

BEGIN RobotArmSimulation

// --- Setup ---

SET L1=30, L2=25, gridSize=30, innerR=5, outerR=55

PROVE all chessboard grid points inside shell [innerR, outerR]

// --- Load waypoints ---

IF waypoints.txt exists THEN load from file

ELSE prompt user interactively

ELSE use demo points

// --- IK for each waypoint ---

FOR each point P in [P0..P4]:

[theta1, theta2, theta3] = InverseKinematics(P.x, P.y, P.z, L1, L2)

STORE angles; PRINT result

ENDFOR

// --- Build trajectory (exclude dead-zone points) ---

trajPoints = points WHERE dist >= innerR AND dist <= outerR

FOR each segment (P_start -> P_end) in trajPoints:

FOR t = 0 to 1 in steps of 1/N:

P(t) = P_start + t * (P_end - P_start)

[a1,a2,a3] = InverseKinematics(P(t))

APPEND P(t) and angles to fullTraj

ENDFOR

ENDFOR

// --- FK Verification ---

FOR each waypoint (except origin):

ee = ForwardKinematics(angles)

error = norm(ee - target)

ASSERT error < 0.001 mm

ENDFOR

// --- PID Control ---

INITIALISE integral=0, prev_error=0

FOR each timestep t in fullTraj:

FOR each joint j = 1,2,3:

e = desired_angle[j] - actual_angle[j]

u = PID(e)  // clamped to +/-400 deg/s

actual_angle[j] += u * dt

packet = format('<J1:a,J2:a,J3:a>')  // serial command

sendMotorCommand(packet)              // UART transmission

ENDFOR

ENDFOR

// --- Plot & Animate ---

PLOT 3D trajectory, joint angles, PID signals, tracking error

FOR each animation frame:

[j0, j1, ee] = ForwardKinematics(ideal_angles[frame])

DRAW Link1(j0->j1), Link2(j1->ee), joints, waypoints

ENDFOR

END RobotArmSimulation

## 6.2 Inverse Kinematics Function

FUNCTION InverseKinematics(x, y, z, L1, L2):

IF x=0 AND y=0 AND z=0 THEN RETURN (0, 0, 0)   // home

theta1 = atan2(y, x)

r      = sqrt(x^2 + y^2)

D      = (r^2 + z^2 - L1^2 - L2^2) / (2*L1*L2)

IF |D| > 1 THEN RAISE 'Unreachable' ENDIF

theta3 = atan2(sqrt(1-D^2), D)                  // elbow-up

theta2 = atan2(z,r) - atan2(L2*sin(t3), L1+L2*cos(t3))

RETURN (theta1, theta2, theta3)

END FUNCTION

## 6.3 PID Controller Function

FUNCTION PID(error, dt, state):

state.integral  += error * dt

derivative       = (error - state.prev_error) / dt

u                = Kp*error + Ki*state.integral + Kd*derivative

u                = clamp(u, -400, +400)         // rate limit

state.prev_error = error

RETURN u   // motor command deg/s

END FUNCTION

# 7. System Block Diagram and Flowchart

7.1 Overall System Block Diagram

Actual

Theta

PID Control Loop (50 Hz)

Motor

u(t)

PID

error

Desired

Theta

Desired Joint

Angles

Inverse

Kinematics

Cartesian

Interpolation

Waypoints

P0..P4

Serial Packet TX

<J1:a, J2:a, J3:a>

(UART / CAN / I2C)

Forward Kinematics

(Animation + FK Verification)

## 7.2 Trajectory Planning Flowchart

START

Load Waypoints

Filter: keep points where inner <= dist <= outerR

Select segment i: P_start to P_end

Set t = 0

Compute P(t) = P_start + t*(P_end – P_start)

Solve IK

Append To fullTraj

Yes

T <= 1?

T = t + 1/N

No

More Segments?

Yes

No

END

# 8. Error Analysis

## 8.1 Error Sources

| Error Type | Source | Simulated? | Magnitude | Impact |
| --- | --- | --- | --- | --- |
| IK Round-Trip Error | Floating-point trig precision | Yes | < 1×10^-6 mm (effectively 0.0000 mm) | None — negligible |
| PID Tracking Lag | Finite Kp; delayed motor response | Yes | 0.1–3 deg peak | Small positional lag |
| Trajectory Discretisation | Finite N=50 steps per segment | Yes | 0 mm (linear) | None — path is exact |
| Encoder Quantisation | Discrete encoder counts in real hardware | No | ~0.1–0.5 deg | Small angle error |
| Motor Backlash | Gear play in real joints | No | 0.5–2 deg | Positioning error |
| Link Flexibility | Elastic deformation under load | No | 0.1–1 mm | Path deviation |
| Comms Delay | UART packet latency | No | 1–5 ms | Phase lag in control |
| Singularity near origin | atan2 undefined at (0,0,z) | Handled | N/A | Special case coded |

## 8.2 Largest Error Source in Simulation

The primary source of error in the simulation is that of PID tracking error. The pure integrator nature of the model ensures that there will always be some degree of error, since the angle of the joint will lag behind the required angle at all times, especially at point changes where the angle of the required line changes abruptly.

Mechanical backlash in the joints is the greatest source of error for a real world application. This involves a situation where the output does not react to any change in the input direction. This is an un-compensated error through PID tuning, and would need pre-loading or backlash compensation.

## 8.3 Trajectory Discretisation Analysis

Since the interpolation itself is entirely linear in Cartesian coordinates, the line IS the mathematical straight line – there is no approximation error involved in the N=50 step resolution. The only potential source of error in the path is the IK round-trip accuracy for each individual step, which has been found to be accurate to 8 decimal places. The stepsizes for each path segment are:

- Segment 1 (P1 to P2): length = 15.81 mm, step = 15.81/50 = 0.316 mm

- Segment 2 (P2 to P3): length = 20.62 mm, step = 20.62/50 = 0.412 mm

- Segment 3 (P3 to P4): length = 15.81 mm, step = 15.81/50 = 0.316 mm

# 9. Reflection

## 9.1 Design Decisions

•	Direct geometric IK approach is preferred over DH parameter modeling because the former provides a non-iterative solution that doesn’t require matrix inversion and has a physical meaning for each angle.

•	The elbow-up variant is used all the time to avoid configuration flipping during motion due to joint discontinuity.

•	The chosen dimensions are L1=30 mm and L2=25 mm as equal length links will result in the formation of an impossible center region. With the difference in 5 mm length, the impossible inner zone of a radius of 5 mm is formed while covering the whole grid.

•	The N=50 interpolations offer a good balance between smoothness of animation and efficiency.

•	The maximum velocity imposed by the PID limiter (400 deg/s) helps avoid instantaneous angle change by a pure-integrator plant when transitioning between waypoints.

## 9.2 Challenges Encountered

•	Origin point (0,0,0) is in the arm dead zone and thus cannot be used as pose tracking. This was solved by detecting the origin point and excluding it from the path while retaining it as reference point in the waypoints array.

•	D values in floating-point form that are only slightly beyond -1 to 1 range at edges of the work space resulted in failure in atan2. A clamping solution was used for dealing with these numerical problems.

•	Precise alignment of animation end effector with trajectory path involved usage of perfect IK angles instead of lagging PID angles for forward kinematics.

## 9.3 Limitations

•	The system model only takes into account kinematics, and does not incorporate any torque, inertia, gravity, or link compliance models.

•	Only elbow up inverse kinematics is considered. In reality, obstacle avoidance could lead to the need for elbow down configuration.

•	The plant model of the PID is a pure integrator, without incorporating actual motor bandwidth limitations, saturation, and back EMF effects.

•	Joint angle restrictions are not incorporated. A real robot has physical joint travel limitations.

•	Collision detection is not included.

## 9.4 Potential Improvements

•	Introduce dynamics simulation by implementing either Newton-Euler equations or the Lagrange equations to account for gravity compensation and inertia feedforward.

•	Use Cartesian space PID controller to achieve a straight path regardless of the lag in joint space.

•	Constrain joint limits (for instance, theta 2 between -90 and 90 degrees).

•	Find solutions for both elbow-up and elbow-down configurations that automatically switch based on obstacles.

•	Use MATLAB’s serialport() object to implement communication with the real serial port instead of simulation.

•	Extend implementation to 6-DOF robot for complete control over orientation.

# 10. Conclusion

This report presents a full simulation of a 3DOF RRR robotic arm moving inside a 30x30x30 mm work environment. The robot arm was built with L1=30mm and L2=25mm, proven to be able to achieve all chessboard grid points based on a reachability analysis.

Inverse kinematics via direct geometric method was done using the elbow-up approach and validated with the forward kinematics round-trip test to obtain a zero positional error result with all four target points: P1=(10,10,25), P2=(25,5,25), P3=(20,25,25), and P4=(5,20,25) mm. Linear interpolation method was used to produce straight-line Cartesian motion path, with 50 steps for each segment.

PID control scheme was employed to simulate the motor driving process with 50Hz sampling rate, and velocity commands sent to the motors in form of serial messages for UART communication. Overall, the simulation results were illustrated using MATLAB 3D animation.

The main drawback of the model is the purely kinematic nature of the simulation. Backlash effect in joint gearbox is certainly the largest source of error, not correctable with the use of PID controller alone.